STATUS REPORT · ML TRACK

Status Report Doc

Tick-to-Trade Engine — ML Track. Progress on the quantized adverse-selection classifier (ml_engineer_brief.md), through roadmap Step 6 of the training/quantization hand-off.

repo: fpga-tick-to-trade-engine   ·   branch: develop   ·   commit: fcea11f   ·   2026-09-09

# What This Is

A two-person portfolio project: an FPGA (Artix-7) datapath takes a synthetic market-data feed to a simulated order at a fixed, measured tick-to-trade latency. A quantized linear classifier sits in the fast path as risk gate 0x09 — it can only flag an order as adversely selected, never decide one on its own; deterministic hard logic always has the final say.

This report covers the ML track only: data, features, labels, training, and quantization, run entirely in Python and headed for an hls4ml/Vitis HLS hand-off. The FPGA/RTL track (parser, book, risk engine, Ethernet) is being built in parallel by the project's other collaborator.

# Headline Numbers

| Quote events (synthetic dataset) | 23,200 |
| --- | --- |
| Rows with a trading signal | 3,681 |
| Train / validation split | 2,541 / 1,140 |
| Positive rate | 52.9% |
| Precision | 0.599 |
| Recall | 0.721 |
| F1 | 0.654 |
| ROC-AUC | 0.569 |

# Checkpoint 1.0 — Foundations

commit 184c9e1   ·   2026-09-01

- config.py — froze the semantic decisions the brief leaves open: window W=16 (current event included), label horizon H=50, dataset seed.

- simulator.py — deterministic synthetic market generator: balanced book, bid/ask depletion, wide spreads, high volatility, spoof-like size changes, bursty/gappy streams.

- ml_golden.py — the fixed-point reference: F0–F7 feature extraction, the y_buy/y_sell label, int8 normalization (floor-shift + saturate, not truncate — the brief's #1 pitfall), the int32 classifier score.

- 9 unit tests, all passing — including a hand-computed 20-event sanity case and a check that the floor-shift matches true Verilog arithmetic-shift semantics on negative numbers.

# Checkpoint 1.1 — Training, and Three Bugs Only Real Output Caught

commit e207750   ·   2026-09-09

Code review alone didn't surface any of these — each one only showed up once train.py actually ran end to end.

| # | Issue | Fix |
| --- | --- | --- |
| 1 | Degenerate label. LABEL_COMBINE="OR" was true on ~97% of rows; all 8 weights rounded to 0, ROC-AUC = 0.500. | Added intended_side(): the master spec's actual FR-35/36 trading rule decides which one-sided label (buy or sell) applies; rows with no signal are dropped. |
| 2 | Too little data once labeling got honest. Side-conditioning left 174 usable rows out of 1,160 — too few to train or evaluate (ROC-AUC = 0.387, i.e. noise). | SCENARIO_REPEATS=20 reruns the scenario set 20x (fresh symbols, same seeded stream) for 3,681 signal rows. |
| 3 | Weight quantization collapsed to zero. Learned float weights were all under 0.22 in magnitude; rounding to nearest int sent every one to 0, again. | Symmetric max-abs scaling: weights + bias scaled together so the largest weight uses the int8 range, before rounding. |
| 4 | Training wasn't actually reproducible. Keras seeds its own weight init from TensorFlow's global RNG, ignoring config.SEED. | tf.keras.utils.set_random_seed(SEED) — verified two full runs now produce byte-identical output. |

Final, on 1,140 held-out rows: F1 0.654 · ROC-AUC 0.569 — vs. a hand-tuned threshold baseline at F1 0.164. Modest but real; reported as a synthetic proxy-label result throughout, not a claim about real markets.

# Files in model/

| File | What it is | Status |
| --- | --- | --- |
| config.py | Every frozen semantic/quantization constant, each documented at the point it's used. | source |
| simulator.py | Deterministic synthetic market-data generator. | source |
| ml_golden.py | Fixed-point reference: features, label, signal rule, normalize, classify. | source |
| metrics.py | Dependency-free precision/recall/F1/ROC-AUC/PR-AUC. | source |
| train.py | Train -> quantize -> re-evaluate in fixed point -> export. | source |
| tests/ (20 tests) | Fixed-point, metrics, and export-consistency tests — all passing. | source |
| model_config.json | Formats, offsets, shifts, weights, thresholds, versions. | exported |
| weights.mem / bias.mem | int8 weights, int32 bias — one value per line. | exported |
| normalization.mem | Per-feature offset/shift pairs for the RTL normalizer. | exported |
| thresholds.mem | Recommended T_high / T_low for the hysteresis gate. | exported |
| golden_vectors.csv | 1,140 x->z->adverse_risk rows for hls4ml bit-exactness checks. | exported |

# Roadmap

1–5.  Freeze definitions, simulator, golden model   done

6.  Train, quantize, export   done

7.  Convert with hls4ml

Backend=Vitis, part xc7a35tfgg484-2, io_parallel — confirm the 32-bit accumulator survives conversion.

8.  Verify bit-exactness

hls_model.predict()/.trace() vs. ml_golden.py on every golden vector — the hard gate before export.

9.  Build and export the IP

Package alongside model_config.json for the FPGA owner.

10.  Integrate and joint-verify

Wire into ml_classifier_wrap.v, run an end-to-end bit-exact soak with the RTL track.